## Domain ISA specification

### 1. Overview & Motivation

Overview & Motivation This ISA is designed for a project that uses cough sounds to help detect TB. In rural areas like Lesotho, many people live far from clinics and can’t afford expensive health checks. But almost everyone has a phone with a microphone. If we can analyze cough sounds quickly and cheaply, people could get early warnings and seek help soon.

Our ISA is meant to run the signal processing part of this idea. It focuses on things like:

- Turning cough sound into numbers the system can understand.

- Comparing the features with stored TB-positive cough samples.

- Making a simple decision like “possible TB” or “Not TB”.

We want the ISA to be simple, use low power, and work well on mobile or embedded devices.

Main design goals:

- Fast and efficient for audio signal processing.

- Low cost and easy to implement.

- Lightweight instruction set that’s easy to decode.

- Supports common operations like array handing math, and branching.

The use of RISC appeared to be a better option over the alternative, being CISC. This comes from the fact that, compared to CISC, RISC has simpler instructions which are easier to decode and execute quickly. Again, it is optimized for pipelining due to uniform instruction size, RISC processors have lower power consumption relative to CISC, which makes them ideal for portable devices, in our case mobile phones.

### 2. Architectural Design Choices

| Feature | Decision | Why |
| --- | --- | --- |
| Instruction Philosophy | RISC | It’s simpler to design and easy to extend later |
| Registers | 16 General-purpose registers(32-bit) and status register | Enough to handle comparisons without wasting space. |
| Data Types | 8-bit, 16-bit, 32-bit | Matches common audio data formats. |
| Addressing | Immediate, register indirect, base and offset | Enough for arrays and memory access. |
| Memory Model | Little endian, word-aligned | Easier to implement in hardware |
| Instruction Format | Fixed 32-bit | Simple do decode and pipeline. R type, I type and B-type. |

### 3. Instruction Set Summary

| Category | Mnemonic(s) | Operands | Description |
| --- | --- | --- | --- |
| Arithmetic | ADD,SUB,MUL,DIV | rd, rs1, rs2 | Perform basic arithmetic operations. |
| Logical | AND,OR,XOR,NOT | rd, rs1, rs2 | Perform bitwise logical operations |
| Data Movement | LD,ST | rd, [rs1+imm] | Load data from or store data to memory |
| Immediate | ADDI,MOVI | rd, imm | Add or move immediate value |
| Branching | BEQ,BNE,BLT,BGT,JMP | rs1, rs2, label | Control program flow based on conditions. |
| Signal Ops | SQR,ABS,CMPF | rd, rs1, rs2 | Square values, take absolute difference, or compare features. |
| Output | OUT | rs1 | Display or send output results |

### 4. Instruction Encoding Summary

### R-Format

| ### Funct7 | ### rs2 | ### rs1 | ### Funct3 | ### rd | ### opcode |
| --- | --- | --- | --- | --- | --- |
| ### 7 bits | ### 5 bits | ### 5 bits | ### 3 bits | ### 5 bits | ### 7 bits |

### I-Format

| ### Imm[11:0] | ### rs1 | ### Funct3 | ### rd | ### opcode |
| --- | --- | --- | --- | --- |
| ### 12 bits | ### 5 bits | ### 3 bits | ### 5 bits | ### 7 bits |

### B-Format

| ### Imm[12] | ### Imm[10:5] | ### rs2 | ### rs1 | ### Funct3 | ### Imm[4:1] | ### Imm[11] | ### opcode |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ### 1 bit | ### 6 bits | ### 5 bits | ### 5 bits | ### 3 bits | ### 4 bits | ### 1 bit | ### 7 bits |

### 5. Design Rationale & Trade-offs

- Simplicity vs Capability:

We chose to use a basic feature extraction method that measures the energy of each signal sample (by squaring the values). This approach keeps the system small, easy to understand, and efficient to run on limited hardware. However, it also means the system has limited capability, it does not capture detailed frequency information like an FFT or MFCC would. The trade-off is that while the program is simpler and faster, it cannot analyze coughs as accurately as more advanced methods.

- Code Density vs Performance:

We used fixed 32-bit instructions, which makes the program code slightly larger in memory. However, this choice simplifies instruction decoding, leading to faster and more consistent execution. The trade-off is between compact code size and higher performance, we prioritized performance and simplicity over code density.

- Hardware Impact:

The system requires a simple datapath that includes an Arithmetic Logic Unit (ALU) capable of performing fast multiplication and addition operations. Since the feature extraction and comparison steps rely on these arithmetic functions, efficient ALU performance is important. Additionally, the system needs quick memory access to store and retrieve cough signal data and reference features smoothly during processing.

- Extensibility:

The system is designed in a way that allows future improvements. More advanced AI-oriented instructions, such as FFT or MFCC extensions, can be added later to enhance its ability to process and analyze sound-based data. This makes the processor adaptable for more complex audio and health-related AI tasks in the future without requiring a complete redesign.