# Objective 1 — Application Analysis (Focus on Cough Detection)

Cough detection is a practical and high-impact application of voice/audio analysis for mobile devices. Instead of recognizing words, the system listens for cough sounds and decides whether a coughing event has occurred. This is useful for health monitoring, early detection of respiratory illness (including in low-resource settings), and for building simple population-level surveillance tools that can run on low-cost phones. For Lesotho and other African countries, a lightweight cough-detection feature can help triage health complaints, support telemedicine, and enable low-cost public-health tools.

### 1.1 Why cough detection is useful in Lesotho and Africa

- Healthcare access: Many people use mobile phones as their main link to health services. A cough-detection feature can let users quickly check if their cough pattern is unusual and suggest seeking care.

- Low-cost screening: In regions where clinical testing is expensive or sparse, a phone-based cough monitor can provide low-cost screening and triage.

- Public health: Aggregated (and anonymized) cough-event data may help detect outbreaks earlier in communities.

- Accessibility: Cough detection does not require literacy or technical skills — it can be used by anyone who can record their cough.

### 1.2 What cough detection does (simple steps)

- Record audio

  - The phone captures a short audio clip from the microphone when the user coughs or when the phone is in passive monitoring mode.

- Preprocessing / cleaning

  - Remove background noise (simple filters) and normalize volume so the detection is stable across environments.

- Feature extraction

  - Convert the audio to a compact representation that highlights the cough’s acoustic patterns. Typical features: short-time energy, zero-crossing rate, spectral features, Mel-frequency cepstral coefficients (MFCCs), or small spectrogram slices.

- Classification

  - A lightweight machine-learning model (e.g., small neural network, random forest, or threshold-based classifier) examines the features and decides whether the audio contains a cough.

- Post-processing & output

  - Optionally, combine multiple detection windows (e.g., majority vote over several seconds) to reduce false positives. Then notify the user or store/send anonymized data for public-health analysis.

### 1.3 Key computational building blocks

Designing the processor requires understanding the math and operations used in these steps:

- Signal processing kernels

  - Short-time Fourier Transform (STFT) or small DFT windows to get frequency content.

  - Windowing (multiply each audio block by a window function).

  - Simple noise filtering (FIR filters or spectral subtraction).

- Feature computations

  - MFCC extraction: a pipeline of FFT/DFT, Mel filterbank sums, log, and discrete cosine transform (DCT).

  - Simpler features: short-time energy, spectral centroid, zero-crossing rate — these are cheaper but may be less accurate.

- Classification kernels

  - Small matrix multiplies, dot products, MAC loops for neural network inference.

  - Activation functions (ReLU, sigmoid) and softmax if needed.

  - Alternatively, simple arithmetic and comparisons for threshold-based detectors.

- Memory & data flow

  - Sliding windows of audio samples; frequent memory reads and writes.

  - Need for small buffers and fast access (cache or on-chip SRAM).

- Math support

  - Trigonometric functions (cos/sin) for transforms or approximations (CORDIC).

  - Log and exp for MFCC (or approximations).

  - Multiply-accumulate (MAC) as a hot spot.

### 1.4 Performance & functional constraints

Real-life deployment on low-cost phones sets important constraints:

- Latency: Detection should be near real-time (a few hundred milliseconds or less), so users get quick feedback.

- Power: The algorithm must be energy-efficient to avoid draining the battery during monitoring.

- Memory: Fit models and buffers into small memory (tens or a few hundred KB on entry-level phones).

- Robustness: The detector must handle background noise, different microphone qualities, and different speaker ages and accents. This often requires preprocessing (noise reduction) or training on diverse datasets.

### 1.5 Design implications for the RISC-V processor

From the building blocks and constraints above, the following processor features are valuable:

- Efficient MAC operations: Add or optimize instructions for multiply-accumulate loops because these appear in FFT/DCT and neural inference.

- Support for fixed-point arithmetic: Fixed-point (e.g., Q15 or Q31) can be far more power- and area-efficient than floating point for small models. Provide integer multiply-accumulate and saturation arithmetic.

- Optional single-precision FPU: If floating-point simplifies algorithms and toolchains, include an F extension—useful for FFTs and DCTs. But ensure a fixed-point fallback.

- Lightweight vector/SIMD support: Even small SIMD instructions that operate on several fixed-point samples at once can speed up feature extraction.

- Fast memory access: A small but fast local SRAM or cache and efficient load/store instructions to support sliding-window buffers.

- Support for fast math primitives: Either library calls for trig/log/exp or small hardware-friendly approximations (CORDIC, lookup tables, polynomial approximations) so transforms and MFCC can run efficiently.

These choices will guide ISA customization (Objective 3) and microarchitecture decisions (Objective 4).

### 1.6 Dataset and model considerations

- Training data: To build reliable cough detectors, you need labeled cough and non-cough audio. Datasets should include noisy real-world examples and local accents/environments. If public datasets are insufficient, collect a small local dataset.

- Model size: Aim for compact models (tiny CNNs or small fully connected nets). Consider on-device quantization (8-bit or 16-bit) to reduce memory and compute.

- Evaluation metrics: Use sensitivity (true positive rate), specificity (true negative rate), and false positive rate per minute/hour for practical evaluation. Also measure CPU cycles, latency, and energy per inference.

### 1.7 How this links to later objectives

- Objective 2 (workload development): The code examples will be small C++ kernels that implement STFT/DFT, MFCC steps, short-time energy, and small inference kernels (dot-product, small matrix multiply, activation). These will serve as benchmarks to compile into RISC-V assembly.

- Objective 3 (ISA customization): Use the workload profile to decide whether to add MAC, saturating arithmetic, small SIMD, or fixed-point helper instructions.

- Objective 4 (microarchitecture): Design a datapath that accelerates the hot loops (MAC, vector ops) while keeping power low. Possibly include a small accelerator for FFT/DCT or a microcoded math unit.

- Objective 5–6 (simulator & testing): Run the C++ kernels and small test models on the simulator, measure performance, and iterate the ISA/microarchitecture.

### 1.8 Summary (simple)

Cough detection is a focused, useful voice-application for mobile health and public health. It requires signal processing (FFT/DFT, MFCC or simpler features), repeated MAC-heavy operations, memory buffers for sliding windows, and simple classifiers. For low-cost phones, fixed-point math, efficient MAC instructions, small memory, and energy-efficient pipeline design are key. Building small C++ kernels that reflect these tasks will allow you to customize a RISC-V ISA and design a microarchitecture that supports cough detection well on low-cost devices.