- Overview & Motivation This ISA is designed for a project that uses cough sounds to help detect TB. In rural areas like Lesotho, many people live far from clinics and can’t afford expensive health checks. But almost everyone has a phone with a microphone. If we can analyze cough sounds quickly and cheaply, people could get early warnings and seek help sooner.

Our ISA is meant to run the signal processing part of this idea. It focuses on things like:

FFT and MFCC feature extraction (turning cough sound into numbers the system can understand),

comparing the features with stored TB-positive cough samples,

making a simple decision like “Possible TB” or “Not TB.”

We want the ISA to be simple, use low power, and work well on mobile or embedded devices.

Main design goals:

Fast and efficient for audio signal processing.

Low cost and easy to implement.

Lightweight instruction set that’s easy to decode.

Supports common operations like array handling, math, and branching.

This workload matches a RISC-style design because RISC has fewer, simpler instructions that can run quickly.

- Architectural Design Choices Feature Decision Why Instruction Philosophy RISC It’s simpler to design and easy to extend later. Registers 16 general-purpose registers (32-bit) + 1 status register Enough to handle FFT, MFCC and comparisons without wasting space. Data Types 8-bit, 16-bit, 32-bit Matches common audio data formats. Addressing Immediate, register indirect, base+offset Enough for arrays and memory access. Memory Model Little endian, word-aligned Easier to implement in hardware. Instruction Format Fixed 32-bit Simple to decode and pipeline. R-type, I-type, and B-type.

Basically, we’re keeping it straightforward but leaving space for future upgrades.

- Instruction Set Summary Category Mnemonic Operands Description Arithmetic ADD, SUB, MUL, DIV rd, rs1, rs2 Basic math Logical AND, OR, XOR, NOT rd, rs1, rs2 Bitwise operations Data Movement LD, ST rd, [rs1+imm] Load/store to memory Immediate ADDI, MOVI rd, imm Add or move immediate value Branching BEQ, BNE, BLT, BGT, JMP rs1, rs2, label Control flow DSP Extensions FFTX, MFCCX, CMPF rd, rs1, rs2 Signal processing Output OUT rs1 Send result out (e.g., display or signal)

The FFTX and MFCCX are special instructions for audio feature extraction, while CMPF helps compare features fast.

- Instruction Encoding Summary R-type (Arithmetic / Logic / DSP)

Copy code | 31-26 | 25-21 | 20-16 | 15-11 | 10-0 | | opcode| rs1 | rs2 | rd | 0 | I-type (Immediate / Load / Store)

pgsql Copy code | 31-26 | 25-21 | 20-16 | 15-0 | | opcode| rs1 | rd | immediate | B-type (Branch)

bash Copy code | 31-26 | 25-21 | 20-16 | 15-0 | | opcode| rs1 | rs2 | offset/label | Opcode: 6 bits

Registers: 5 bits each

Immediate/Offset: 16 bits

This structure is easy to decode, which is perfect for our small processor.

- Design Rationale & Trade-offs We kept the design as minimal as possible while still being useful for our specific workload.

Simplicity vs Capability: We didn’t add too many fancy instructions to keep it small, but we included FFTX, MFCCX, and CMPF because those are key for cough analysis.

Code Density vs Performance: Using fixed 32-bit instructions makes the code a bit longer, but decoding is way easier and faster.

Hardware Impact: A regular instruction format means simpler control logic. Special DSP instructions will need some extra hardware, but it’s manageable.

Extensibility: We left space in the opcode field for future instructions (like vector or SIMD ops if we want to go further).

References Domain Analysis pt1 (Design Masters Project)

RISC-V International, 2019

WHO TB Report 2023

Botha et al., 2018; Larson et al., 2011