NAMES: MOKOTOI TLOTLISO

STUDENT #: 202321234

DATE: 12/11/2025

EMI-3 ACTIVE FILTERS

Title: Active Filters

Abstract:

This laboratory report examines the practical performance of second-order Sallen-Key active filters, with a particular focus on low-pass and high-pass Butterworth implementations. The experiment aimed to characterize their frequency response by measuring voltage gain over a specified range of frequencies. The experimental data closely matched theoretical predictions, with both the passband gain and the cut-off frequency showing strong agreement with calculated values. The filters demonstrated the expected Butterworth behaviour, featuring a smooth roll-off of about –40 dB per decade. Overall, the findings highlight the influence of component choices and circuit configuration on filter behaviour and frequency response.

Introduction:

In electronics, the ability to selectively filter specific frequency components from a signal is essential. Applications range from reducing noise in audio systems to separating communication channels in radio networks. Although passive filters made from resistors and capacitors can perform basic filtering, they suffer from several limitations, such as signal attenuation and the inability to provide gain.

The introduction of operational amplifiers (op-amps) enabled the development of active filters, which overcome these limitations. Active filters offer signal amplification, high input impedance, low output impedance, and more accurate control of frequency characteristics, including sharper roll-off rates. Among the many active filter topologies, the Sallen-Key configuration is widely favored for its simplicity, stability, and ease of design.

This report focuses on the analysis of Sallen-Key active filters, specifically second-order low-pass and high-pass filters designed to exhibit a Butterworth response. The Butterworth characteristic is valued for its maximally flat passband, ensuring minimal ripple and accurate transmission of desired frequencies. The objective of this experiment is to construct these filters and analyze their gain–frequency response. Through this investigation, we aim to confirm key performance parameters, such as the passband gain, cut-off frequency, and the theoretical roll-off rate of –40 dB per decade for second-order filters.

Theory:

Active filters are electronic circuits that use a combination of passive components, such as resistors and capacitors, together with active elements like operational amplifiers. This integration allows the circuit not only to selectively pass desired frequency ranges but also to provide amplification. Active filters are generally categorized based on the portion of the frequency spectrum they allow to pass: low-pass, high-pass, band-pass, and band-stop filters.

The Butterworth Response
The Butterworth characteristic is widely used because it provides a maximally flat passband, meaning the gain remains smooth and ripple-free up to the cut-off frequency. This makes Butterworth filters ideal for applications where signal fidelity and stability are more critical than achieving an extremely sharp transition.

For a second-order Sallen-Key filter, the passband gain is defined by:

Av=1+RfRg

​​

where Rf and ​Rg are the feedback and grounding resistors of the non-inverting amplifier.
Note: To achieve the Butterworth alignment, the gain must be approximately 1.586:

Av=1+RfRg=1.586→RfRg=0.586

### Filter Parameters and Key Equations

Passband Gain:
This is the constant gain provided by the op-amp within the passband and is given by:

Av=1+RfRg

Cut-off Frequency:
For second-order Sallen-Key low-pass and high-pass filters, the cut-off frequency is determined using the basic RC relationship:

fc=12πRC

This corresponds to the -3 dB point, where the output power is half of the passband power and the output voltage drops to 70.7% of its maximum value.

Roll-off and Filter Order:
The “order” of a filter depends on the number of reactive components (poles) it contains, which directly determines the steepness of its roll-off.

- A first-order filter rolls off at –20 dB/decade.

- A second-order filter rolls off at –40 dB/decade.

For a low-pass filter, this means that beyond the cut-off frequency, the gain decreases by 40 dB for every tenfold increase in frequency.

Quality Factor and Bandwidth:
The quality factor QQQ describes the damping of the filter. High-Q filters show a noticeable peak near the cut-off frequency. For a Butterworth response, Q=12=0.707 , ensuring a completely flat passband with no peaking.
For a low-pass filter, the bandwidth is approximately equal to the cut-off frequency.

### Filter Variants

Low-Pass Configuration:
In this design, resistors form the input path while capacitors are connected to ground, creating two RC low-pass sections. Low-frequency signals pass with the designed gain, while high-frequency components are diverted to ground through the capacitors. This results in a stable low-frequency passband and a smooth roll-off after the cut-off frequency.

High-Pass Configuration:
The high-pass version reverses the roles of resistors and capacitors. Capacitors form the signal path and resistors provide the grounding path. Low-frequency signals are blocked because they cannot pass through the capacitors, whereas high-frequency signals are allowed through with amplification. The resulting response mirrors the low-pass behavior, but inverted around the cut-off frequency—maintaining stable gain above fc​ and attenuating signals below it.

This theoretical background forms the basis for predicting expected filter performance, which will be compared with experimental results to validate the principles of active filter design.

Experimental Results:

- Curve Plot:

2.

Av=1+R1R2

Given:
R1=5.86kΩ and R1=5.86kΩ

Av=1+105.86=2.706

Converting this gain to decibels:

AvdB=20log10Av

AvdB=20log102.706

AvdB=8.65

- Calculate the -3 dB level:

7.241dB -3dB =4.241dB

The gain is at 4.241dB    
From the plotted response curve, this gain is reached at around 6 kHz, marked with the orange circle on the graph.

- Cut-off Frequency Calculation

fc=12πRC

Given:
 R=3000Ω
 C=0.001μF=1×10-9μF

fc=12π×30000×0.001×10-9=5305.16Hz

fc=5.305Hz

The measured cut-off frequency was 6.678 kHz, which is close to the theoretical value.

5.The measured cut-off frequency is 6.678kHz

10×6kHz=60kHz

The closest point 70kHz.

The gain at 70kHz is -33.11dB. Marked with the grey diamond on curve.

Part 2

- Plot:

Av=1+R1R2

Where: R1=5.86kΩ and R1=5.86kΩ

So Av=1+105.86=2.706

We convert this gain to decibels

AvdB=20log10Av

AvdB=20log102.706

AvdB=8.65

- Calculate the -3 dB level:

4.005dB -3dB =1.005dB

The gain is at 1.005dB

From the curve plot, the gain is at the cut-off frequency 5.5kHz. Marked with orange circle on graph.

- Cut-off frequency

fc=12πRC

WhereR=30000Ω, C=0.001μF=1×10-9μF

fc=12π×30000×0.001×10-9=5305.16Hz

fc=5.305Hz

- The measured cut-off frequency is 5.5kHz

0.10×5.5kHz=0.55kHz

The closest point 500Hz.

The gain at 50Hz is -36.695dB. Marked with grey diamond on graph.

- Plot:

4.005dB -3dB =1.004dB

The gain is at 1.005dB

The op-amp runs out of speed. The LM741 is too slow to maintain gain at MHz frequencies, so even the high-pass filter's output eventually falls off.

Discussion:

The experimental results clearly demonstrated the operating principles of second-order Sallen-Key active filters and confirmed several key theoretical predictions, while also revealing deviations caused by component tolerances and non-ideal op-amp behavior. The main objective—characterizing the frequency response of the low-pass and high-pass filters—was successfully achieved. Both configurations produced gain-frequency curves consistent with a second-order Butterworth response, and the expected roll-off rate of approximately –40 dB/decade was verified.

For the low-pass filter, the gain dropped from 7.241 dB in the passband to –33.11 dB at 70 kHz, which is close to one decade above its measured cut-off frequency of 6 kHz. This represents a decrease of roughly 40.35 dB, closely matching the theoretical roll-off. The high-pass filter likewise demonstrated strong attenuation below its cut-off frequency, showing a gain of –36.70 dB at 500 Hz—approximately one decade below the measured 5.5 kHz cut-off.

Despite this agreement, a notable discrepancy emerged between the theoretical and measured passband gains. The expected gain of 2.706 (8.65 dB) was not achieved in either filter. The low-pass filter reached only 7.241 dB, and the high-pass filter measured 4.005 dB before declining. This consistent reduction in gain can be attributed to the finite gain-bandwidth product (GBP) of the LM741 op-amp. The theoretical calculations assume an ideal op-amp with infinite open-loop gain, whereas the LM741—having a typical GBP of about 1 MHz—experiences a decline in open-loop gain as frequency increases. This loss directly reduces the achievable closed-loop gain, with the effect being more pronounced in the high-pass filter where gain is measured at higher frequencies.

The measured cut-off frequencies also differed from the calculated value. The theoretical cut-off frequency,

fc=12πRC=12π(3000)(10-9)=5.305 kHz,

was slightly lower than the measured values of 6 kHz (LPF) and 5.5 kHz (HPF). The upward shift—13.1% for the LPF and 3.7% for the HPF—is consistent with what would be expected from ±5% or ±10% tolerance components. A decrease in actual resistance or capacitance values relative to their nominal ratings would naturally increase the cut-off frequency, explaining the observed differences.

Further observations of the high-pass filter at very high frequencies (up to 50 MHz) revealed the presence of an upper cut-off frequency. This effect is not predicted by the ideal transfer function but arises from the limited bandwidth of the LM741 op-amp. As the input frequency approaches or exceeds the op-amp’s GBP, the device cannot maintain a stable closed-loop gain, causing the output amplitude to drop. This behaviour effectively gives the filter a band-pass characteristic at very high frequencies, highlighting limitations imposed by the op-amp’s finite slew rate and bandwidth.

In summary, the experiment verified the theoretical behaviour of second-order Butterworth filters, including the expected cut-off region and –40 dB/decade roll-off. The deviations observed in gain and cut-off frequency are fully consistent with real-world factors such as component tolerances and op-amp performance constraints. Rather than contradicting theory, these results provide valuable insight into the practical limitations of the LM741 and active filter design in general.

Conclusion:

This laboratory investigation successfully characterized the frequency response of second-order Sallen-Key active filters. The gain-frequency curves for both the low-pass and high-pass designs confirmed the expected second-order behaviour, demonstrating a roll-off rate close to –40 dB per decade. The measured cut-off frequencies of 6.0 kHz (low-pass) and 5.5 kHz (high-pass) showed good agreement with the theoretical value of 5.3 kHz, with the small differences explained by normal component tolerances.

A notable observation was the deviation between the measured and theoretical passband gains. Instead of the predicted 8.65 dB, the low-pass filter achieved 7.24 dB, while the high-pass filter reached only 4.01 dB. This consistent reduction, together with the appearance of an upper cut-off frequency in the high-pass filter’s extended response, highlights the practical limitations of the LM741 op-amp—particularly its finite gain-bandwidth product. Overall, the experiment confirmed the theoretical behaviour of the filters while clearly demonstrating how non-ideal op-amp characteristics influence real-world circuit performance.

References:

- [1] A. S. Sedra and K. C. Smith, Microelectronic Circuits, 7th ed. New York, NY, USA: Oxford University Press, 2014.