National University of Lesotho

Department of Physics and Electronics

EE3404: Classroom Questions and Assignment Questions

Classroom Questions

- Conceptual: Why can capacitors and inductors be considered “memory” elements in circuits? How does this property affect circuit behavior over time?

- Application: Your phone uses a power supply with capacitors inside. What role do these capacitors play in ensuring the phone runs smoothly?

- Conceptual: If we double the area of a capacitor’s plates while keeping distance constant, what happens to its capacitance and why?

- Application: Inductors are widely used in car ignition systems. Why do you think an inductor’s ability to store magnetic energy makes it suitable for this application?

- Problem-solving: If an inductor has L=10mH, and the current through it changes at a rate of 200A/s, what is the voltage across it?

- Conceptual: What is the difference between independent and dependent sources? Can you think of a real-world situation where a dependent source might appear?

- Application: Radio tuning circuits rely on capacitors and inductors. Which properties of these elements make them useful for selecting specific frequencies?

- Problem-solving: Two resistors R1=2kΩ and R2=3kΩ are in series with a 10V source. Use the voltage divider rule to calculate the voltage across each resistor.

- Conceptual: Why must we “zero out” other independent sources when applying the principle of superposition?

- Challenge: Imagine you have two independent voltage sources and one dependent source in a circuit. Would the superposition theorem still apply directly? Why or why not?

Assignment Questions

- Calculation (Capacitance Design): A capacitor is constructed using two square metal plates, each of side length 10 cm, separated by 1 mm of air. Calculate its capacitance. How would the capacitance change if the separation was halved?

- Derivation & Analysis (Inductor Energy): Derive the energy stored in an inductor starting from v=Ldtdi. Then calculate the energy stored when a 5 mH inductor carries 2 A of current.

- Circuit Analysis (Superposition with Real-World Application): In a household circuit, assume you have a 12V DC supply and a 10V battery both connected with resistors. Using superposition, calculate the current in a 5Ω resistor. Clearly show each step.

- Design Challenge (Voltage Divider): You are tasked with designing a simple voltage divider to step down a 12V car battery to 5V to power a small sensor. Choose appropriate resistor values and justify your choice, considering power dissipation and efficiency.

- Research/Conceptual (Real-World Application): Dependent sources are widely used in transistor amplifier models. Write a short technical note (2–3 pages) explaining how dependent sources represent transistor behavior. Include diagrams and equations.